Case 1-A

Geometrical description of the site (see fig. 1).

This is a flat land, covered by grass, 10 cm thick, over which there is an indefinite straight road elevated 2m above the ground plane. The road has 2 lanes, and the road surface is 11m wide, paved with smooth asphaltum (not sound absorbing).

- Ground factor of the road paving G = 0 (see ISO/DIS 9613/2 clause 7.3.1): it means an absorption coefficient equal to zero, or a reflection coefficient of 1, or an infinite real impedance at all the frequencies.

- Ground factor of the soft ground covered by grass G = 1. If impedance data are required, these can be assumed to be obtainable by the following formulas [1]:

ReZ=ρ⋅c⋅1+9.08⋅fs−0.75ImZ=-ρ⋅c⋅11.9⋅fs−0.73

Where  is given the numerical values of 300 s-1,  f is the frequency and c is the air impedance.

There is a building, measuring m 20x20x10(height), made of a material with an average reflection coefficient =0.8 (that is, Alpha=0.2) at any frequency. The facade of the building is located at 100.5 m from the centre of the road.

There are 6 receiver positions, located on the simmetry axis of the building:

1) 25m from the centre of the road, h=2m over the ground plane

2) 50m from the centre of the road, h=2m over the ground plane

3) 100m from the centre of the road, h=2m over the ground plane, 0.5m in front of the building

4) 100m from the centre of the road, h=5m over the ground plane, 0.5m in front of the building

5) 100m from the centre of the road, h=8m over the ground plane, 0.5m in front of the building

6) 125m from the centre of the road, h=2m over the ground plane, 4.5m behind of the building

Fig. 1

Traffic Flow Data

Flow of light vehicles QL=1000 vehicles/hour

Flow of heavy vehicles QH=200 vehicles/hour

Speed of light vehicles VL=100 km/h

Speed of heavy vehicles VH=80 km/h

These data are total for both lanes, being the traffic flow equally divided in both directions.

Meteorological conditions and propagation data

- Wind speed = 0 m/s

- Air temperature = 20 °C

- Air pressure = 101.325 kPa

- No temperature gradient inversion

[1]  Chessel C.I. - “propagation of noise along a finite impedance boundary” - J.A.S.A. 62, 825-834 (1977)

Case 1-B

This case is equal to the previous one, but a screen has been inserted on the side of the road near the building, having indefinite length and an height of 3m. The screen is very heavy, and sound reflecting (=1, Alpha =0 at any frequency). See fig. 2.

Fig. 2

Fig. 3

Results of the simulation

Identification

Name: .....................................................................................................................................

Company: .....................................................................................................................................

Address: .....................................................................................................................................

Tel: ............................................. Fax: .................................................

Name and version of the programme.............................................................................

RESULTS

CASE 1-A (without screen)

| Position | 125 Hz | 250 Hz | 500 Hz | 1 kHz | 2 kHz | 4 kHz | “A” |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |

CASE 1-B (with screen)

| Position | 125 Hz | 250 Hz | 500 Hz | 1 kHz | 2 kHz | 4 kHz | “A” |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |

Observations and remarks :

Send this sheet, any other information to explain the results, and if possible a diskette containing the above data in a readable form (ASCII, Excel, Qpro, Lotus, etc.) to the following address before 30 April 1995:

Prof. Pompoli Roberto

Istituto di Ingegneria

Via G. Saragat

44100 FERRARA (Italy)

fax (until 15 march 1995) : +39 532 740983

(after 15 march 1995) : to be communicated...