# Applied Acoustics  (Engineering) – 11/02/2013

Surname and Name:

A

B

C

D

E

F

Matricula:

1) Make the energetic sum of the following two values in dB: 80+E and 78+F.

Ltot =                                 dB

2) Compute the total sound pressure level of the following octave band spectrum:

| f (Hz) | 63 | 125 | 250 | 500 | 1000 | 2000 | 4000 | 8000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lp (dB) | 86+A | 82+B | 78+C | 77+D | 76+E | 74+F | 75+D | 79+A |

Ltot  =                                 dB

3) Compute the total A-weighted sound pressure level of the spectrum shown above.

Ltot,A =		      	      dB(A)

4) The average sound pressure level is measured over a rectangular surface, surrounding a noise source. The measurement surface is (5+D) meters long, (3+E) meters wide and (2+F) meters tall, and is located over a reflecting plane, in free field (outdoors).

2+F

3+E

5+D

Averaging over the 5 free faces of the rectangular surface, a sound pressure level Lp,avg of 70+D dB(A) is measured.  Compute the sound power level of the noise source.

Lw = 			      dB(A)

5) e 6) In a construction site, a worker is exposed for 4 + A hours to the noise of a machine having a sound power level Lw=100+C dB(A), at a distance of 2+F meters. The machine is installed over a reflecting surface, outdoors.

Compute the following values:

Sound pressure level at the workplace					Leq = 			      dB(A)

Personal exposure level of the worker					Lep = 			      dB(A)

Note: each correct answer gives 5 points (tolerance: +/- 0.5 dB).

Total score of exercises:

Oral examination:

TOTAL SCORE   Points: