# Applied Acoustics  (Engineering) – 26/07/2013

Surname and Name:

A

B

C

D

E

F

Matricula:

1) Two machines create narrowband noise, the first one in the octave band of 125 Hz, and the second one in the octave band of 2000 Hz, respectively of 80+E and 78+F dB. Compute the total SPL in dB (LIN, unweighted) when they operate simultaneously.

Ltot =                                 dB

2) As the previous exercise, but compute the total SPL in dB(A) (A-weighted)

Ltot =                             dB(A)

3) An average sound pressure level is measured inside a rectangular reverberant room, containing a noise source. The room is (15+D) meters long, (8+E) meters wide and (2+F) meters tall.

2+F

8+E

15+D

The reverberation time of the room is 4+F/5 s.

The average sound pressure level inside the room is Lp,avg = 70+D dB(A).  Compute the sound power level of the noise source.

Lw = 			      dB(A)

4) and 5) In a construction site, a worker is exposed to the following noise profile:

| 2 + E/2 hours at 70+F dB(A) | 2+C/2 hours at 75+D dB(A) | 2 hour at 74 dB(A) |
| --- | --- | --- |

Compute the following values:

Average sound pressure level at the workplace				Leq = 			      dB(A)

Personal exposure level of the worker					Lep = 			      dB(A)

6) and 7) In a standing wave tube, the ratio between the maximum pressure and the minimum pressure is 2+F/10. Compute the absorption coefficient of the sample and the energetic ratio re=I/Dc.

Absorption coefficient of the sample						 =

Energetic ratio	 re=I/Dc							re =

Note: each correct answer gives 5 points (tolerance: +/- 0.5 dB, +/- 10%).

Total score of exercises:

Oral examination:

TOTAL SCORE   Points: