# Applied Acoustics – 05/09/2014

Cognome e Nome:

A

B

C

D

E

F

Matricola:

1) Make the energetic sum of the following two values in dB: 80+E and 78+F.

Ltot =                                 dB

2) Compute the total sound pressure level of the following octave band spectrum:

| f (Hz) | 63 | 125 | 250 | 500 | 1000 | 2000 | 4000 | 8000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lp (dB) | 86+A | 82+B | 78+C | 77+D | 76+E | 74+F | 75+D | 79+A |

Ltot  =                                 dB

3) Compute the total A-weighted sound pressure level of the spectrum shown above.

Ltot,A =		      	      dB(A)

4) Inside a building having a volume of 1000 m3, a sound pressure level Lp1=80+E dB(A) is measured, being caused by a number of machines, producing a perfect diffuse field. The reverberation time T1 of the room is equal to 3+F s. An environmental correction is installed based on the introduction of 300+EF m2 of “baffles”, having an absorption coefficient  = 0.75. Compute the new values of the reverberation time and of the sound pressure level after the treatment.

- T2 =                                	s

- Lp2 = 				dB(A)

5) A brick wall is separating two rooms. The wall has a surface of 10+C m2, weighting 200+EF kg/m2. In the first room a noise source is creating a sound pressure level Lp1 = 80+D dB at a frequency of 500 Hz. The second room has a volume V2 = 100+CD m3, and a reverberation tome T2 = 1+F/10 s. Compute the sound pressure level Lp2 in the second room.

- Lp2 =                              	dB

VOTO FINALE   Punti: