# Applied Acoustics – 07/09/2015

Cognome e Nome:

A

B

C

D

E

F

Matricola:

- A white noise is covering 10 octave bands (31.5 Hz to 16 kHz) and in the first band the SPL is equal to 70+D dB. Compute the SPL of the last band and the total “A-weighted” sound pressure level:

- L16kHz =     	                           dB				2 points

- Ltot,A =                             	   dB(A)			3 points

2) A sound source radiates a pure tone at the frequency of 25+F*1000 Hz, having a SPL of 90+C dB. Compute the total unweighted SPL and the total “A-weighted” SPL, employing at least 1/3 octave band accuracy:

- Ltot,L =			   	   dB				2 points

- Ltot,A =			   	   dB(A)			3 points

3) A noise source, radiating wide-band noise, is located over a reflecting surface, outdoors. The source is surrounded by an enveloping measurement surface having an area S=100+EF m2. The average sound pressure level of 70+D dB(A) is measured moving the microphone aver the surface. Compute the sound power level Lw of the source.

- Lw = 				   dB(A)			5 points

4) The same source of exercise 3) is now located inside a building having an environmental correction factor K2=1+E/2 dB(A). Compute the new average value of SPL which is measured on the measurement surface.

- Lp = 				   dB(A)			5 points

5) Inside a building having a volume of 1000 m3, the reverberation time T1 of the room is equal to 3+F s. An environmental correction is installed causing the reverberation time T2 to reduce to 1/3 of the original value. Compute the new value of the equivalent absorption area A2 and the reduction L of the sound pressure level after the treatment.

- A2 =                                	s 				2 points

- L = 				dB				3 points

6) A brick wall is separating two rooms. The wall has a surface of 10+C m2, weighting 200+EF kg/m2. In the first room a noise source is creating a sound pressure level Lp1 = 80+D dB at a frequency of 500 Hz. The second room has a volume V2 = 100+CD m3, and an absorption area A2 of 10+E m2. Compute the sound pressure level Lp2 in the second room.

- Lp2 =                              	dB				5 points

FINAL SCORE   Points: