# Applied Acoustics – 10 July 2015

Name & Surname:

A

B

C

D

E

F

Date of Birth:

Exercise 1 (tolerance +/- 0.5 dB)

At the entrance of an infinite pipe, a circular loudspeaker with diameter of 100mm is radiating a plane, progressive wave with a power of (10+F)/100 W.

The air density is =E/10 +1  kg/m3. The air temperature is  t=10+D °C.

Determine the Sound Intensity in W/m2, the sound pressure in Pa, and the particle velocity in mm/s.

- Sound Intensity			                 		W/m2       	(4 points)

- Sound Pressure		      		                    	Pa	       	(4 points)

- Particle Velocity 		     		                        mm/s	       	(4 points)

Exercise 2 (tolerance +/- 0.5 dB)

R

S

C+10

10+E

F/10+1

An omnidirectional point source is located above an absorbing ground (α=1), at an height of F/10+1 m. The Sound Power Level Lw is equal to 100+D dB.

A microphone is located at an horizontal distance of 10+E m, and at an height of C+10 m above the absorbing ground.

Determine the values of the SPL at the microphone.

- SPL				                 		dB       		(6 points)

Exercise 3 (tolerance +/- 0.5 dB)

Inside a reverberant room having a volume V equal to 500+EF m3 we have a sound source, of known power level in each octave band. Also the value of reverberation time T20, is known in each octave band. Compute the total wide-band sound pressure level (SPL) in dB, and the total SPL with “A” weighting in dB(A).

| freq. (Hz) | 125 | 250 | 500 | 1000 | 2000 | 4000 |
| --- | --- | --- | --- | --- | --- | --- |
| LW (dB) | 84+F | 80+E | 75+D | 70+C | 70+B | 70+A |
| T20 (s) | 5+F/5 | 4+E/8 | 3+D/10 | 3+C/10 | 2+B/10 | 1+A/10 |

- Total sound pressure level	                 		dB       		(6 points)

- Total A-weighted SPL      		                    	dB(A)	       	(6 points)