# Applied Acoustics – 21 April 2017

Name & Surname:

A

B

C

D

E

F

Date of Birth:

Exercise 1 (tolerance +/- 0.5 dB)

Inside a standing wave tube an intensimetric measurement is performed, and the following values are found: Sound Pressure Level is equal to 80+F dB, Particle Velocity Level is equal to 80+E dB, and the phase shift between pressure and velocity is 70+D degrees. Compute the Sound Intensity Level, the Energy Density Level and the Reactivity Index (RI=Ld-Li)

- Sound Intensity Level Li			                 	dB	      	(4 points)

- Energy Density Level Ld		      		             dB	       	(3 points)

- Reactivity Index RI 		     		             dB	       	(3 points)

Exercise 2 (tolerance +/- 0.5 dB)

An omnidirectional spherical point source, with a radius of 40+E mm, generates spherical waves in free field. The radial velocity of the surface of the sphere is 10+F mm/s, at a frequency of 159.2 Hz.

A pressure-velocity microphone probe is located at a distance of 1+D/10 m from the center of the sphere.

Determine the following values at the microphone.

- Sound Pressure Level					            dB       	(4 points)

- Particle Velocity Level				                 	dB       	(3 points)

- Sound Intensity Level			                 		dB       	(3 points)

Exercise 3 (tolerance +/- 100 Hz)

A pure tone at the frequency of 28+F kHz is sampled employing an analog-to-digital converter NOT equipped with a proper anti-aliasing filter, and operating at a sampling frequency of 48 kHz.

Draw the spectrum of the digitally-sampled waveform obtained with an FFT of 8192 points and Rectangular window, specify the frequency of the peak and the Nyquist frequency.

- Peak frequency						            Hz       	(4 points)

- Nyquist frequency							Hz	(3 points)

SPL (dB)

Chart	(3 points)

fNyquist	f (kHz)